Context

In the context of Visible Light Positioning (VLP), existing methods largely fall into two categories: model-based and data-driven. Model-based approaches typically require minimal calibration and have low computational complexity, making them suitable for real-time execution. Their main limitation is the reliance on channel equations that assume a Lambertian source of order m. Although the LED chip may be approximately Lambertian, practical factors—manufacturing variance, lenses, slight tilts, decentered optical axes, fixture geometry, among others—often lead to luminous intensity patterns that deviate from the ideal Lambertian model and may even vary across lots of the same LED model. These deviations limit sub-centimeter accuracy.

Data-driven approaches, in contrast, learn the effective radiation pattern and environment-specific channel characteristics during training, making them promising candidates for sub-centimeter positioning. However, they generally demand large training datasets to be reliable, which constrains deployment beyond laboratory settings, and trained models may not transfer well to different LED units.

Within this context, we outline the following potential directions.

Proposed Directions

Proposal P1: Estimating the LED Irradiation Pattern: Calibration and ML-Based Exploration.

Description: The current state of the art (SOTA) largely trusts the analytical channel model. Prior evidence suggests that “truthing” the model can improve accuracy. We propose to fuse two data sources—(i) the datasheet intensity profile and (ii) minimally calibrated in-situ measurements—to estimate the pre-/post-experimental irradiation pattern. We will compare classical model-based VLP (e.g., triangulation) against an estimator that explicitly uses the learned irradiation pattern.

Proposal P1.2 : Physics-Informed Neural Networks (PINNs) for LED Pattern Calibration.

Description: We train a PINN that learns from the datasheet intensity distribution and adapts with a small number of in-situ samples to calibrate the LED’s irradiation pattern. This yields a pattern model that is more faithful than generic analytic forms and readily generalizes across different LED models. The resulting pattern model can be used directly for position estimation and compared against (i) plain neural networks and (ii) classical model-based baselines.

Proposal P2 : 3D Position Estimation via Machine Learning in a Beam-Steering OWP System.

Description: Regardless of prior knowledge of the LED pattern, we consider a specific LED (e.g., LZ1) and collect a large dataset using the set of orientations introduced in the referenced TCOM work. The goal is robust 3D positioning using machine learning within an OWP system with beam steering. Different architectures will be investigated to develop a system capable of sub-centimeter positioning accuracy.

Proposal P3 : Experimental Validation of the TCOM Paper in 3D (Model-Based).

Description: We reproduce and extend the TCOM methodology to 3D, validating the model-based pipeline experimentally and quantifying accuracy versus ideal Lambertian assumptions.

Proposal P4 : Cooperative Fusion of OWC and OWP (OWC-assisted by OWP).

Description: To investigate and validate the performance improvement in communication (SNR, throughput) of a beam-steering OWC system assisted by an OWP subsystem, compared to a conventional static OWC system (i.e., one not assisted by OWP). The architecture comprises two separate subsystems (OWC and OWP) that can operate independently and are self-contained; in cooperative mode, however, OWP assists OWC by supplying position estimates via time-division multiplexing (TDM). To enlarge spatial coverage, OWP performs systematic scanning over predefined regions.

Proposal P5 : Multi beamstearring APs for OWP and OWC: Oportunities, Challenges and Applications.

Description: This work will explore into the posibilities of use multi-beamstearing APs from a theoretical perspective and a experimental.

Engineering Stuffs

RX PCB : Desarrollo de un PCB para el PBX61 confiable.

TX PCB : Desarrollo dos PCB, uno para solo el LED y otro para el DRIVER.

CALIBRACION : Mediante un puntero laser se debe CONFIRMAR que el sistema esta apuntando en la dirección correcta.

ROBOT MOVIMIENTOS : Se debe de estudiar como se comportan los movimientos del ROBOT y entre que rango es fiable tenerlo. Además ver si se puede disminuir el TAMAÑO.

P1.2

Timeline

P3

P1

P4

P2